使用说明书

安全使用建议

- 本控制器为6V/12V自动适应，首次安装时，请确保电池有足够的电压，以便控制器能够识别为正确的电池类型。

- 将控制器尽量靠近电池安装，以避免电线过长造成压降，影响正常电压判断。

- 本控制器只适用于各种铅酸电池（包括开口，密封，胶体等），请勿使用于其他电池（包括锂电池，镍氢电池等）。

- 本控制器只能使用光伏板作为充电源，请勿使用直流或其他电源作为充电源。

- 本控制器运行的时候会发热，请注意将控制器安装在平整，通风良好的表面。

- 产品特点

- 采用工业级主控芯片。

- LED显示，断电记忆功能，IP57防护等级。

- 3.完整的四阶段PWM充电管理。

- 4.内置过流/短路保护，开路保护，反接保护，均为自恢复型，不损伤控制器。

- 5.双MOS防倒灌电路，超低发热量。

- 系统连接

- 将蓄电池正负极按图示接入控制器，控制器将会自动检测蓄电池电压，并依据检测到的电池电压进行系统类型识别。

- 将负载正负极按图示接入控制器，主要不要反接。

- 将太阳能板按图示接入控制器。

- 注意：请严格按照以上顺序进行接入，否则可能会损坏控制器。

- 拆卸顺序与接线顺序相反。

- 运行模式

- 本系列控制器的运行模式为出厂内置，详细的运行模式请与当地经销商联系。

- 控制器内置的运行模式可能为以下任意一种：

| - 模式 | - 运行模式 |
| --- | --- |
| - 光控+时控 | - 光控打开负载+1-13小时延时关闭 |
| - 纯光控 | - 天黑打开负载，天亮关闭负载（有延时） |
| - 系统模式 | - 负载24小时打开（除非电池低电保护） |

- 故障指南

| - 异常现象 | - 可能原因 | - 解决办法 |
| --- | --- | --- |
| - 阳光充足，充电指示灯不亮 | - 光伏板开路或反接 | - 重新连接好光伏板 |
| - 负载指示灯不亮 | - 模式设置错误 | - 重新设置 |
|  | - 电池电压太低 | - 重新充电 |
| - 负载指示灯慢闪 | - 负载过流 | - 减小负载功率 |
| - 负载指示灯快闪 | - 短路保护 | - 移除短路，自动恢复 |
| - 控制器不亮 | - 电池电压太低/反接 | - 更换电池/检查反接 |

- 技术参数

| 型号 | 5A-12V 系统模式 | 5A-12V-L 纯光控模式 | 5A-12V-8H 光控+时控 |
| --- | --- | --- | --- |
| 系统电压 | 6V/12V 自适应 |  |  |
| 额定充电电流 | 5A |  |  |
| 额定放电电流 | 5A |  |  |
| 最高光伏电压 | 22V |  |  |
| 充电回路压降 | <0.2V |  |  |
| 放电回路压降 | <0.25V |  |  |
| 提升充电压 | 14.6V |  |  |
| 直充电压 | 14.5V |  |  |
| 恒压充电压 | 14.2V PWM |  |  |
| 浮充电压 | 13.8V |  |  |
| 充电恢复电压 | 13.0V |  |  |
| 放电截止电压 | 11.2V |  |  |
| 放电恢复电压 | 12.6V |  |  |
| 待机电流 | <10mA |  |  |
| 光控开灯阀值 | 光伏板8V（光控开灯延时10s） |  |  |
| 光控关灯阀值 | 光伏板8V（光控关灯延时60s） |  |  |
| 温度补偿 | N/C |  |  |
| 工作温度 | -35~+60 ℃ |  |  |
| 尺寸/重量 | 60x25x18mm/50g |  |  |

*红色字体标记电压仅对应12V系统，如使用6V系统，请/2.

*产品规格如有更改，怒不另行通知。

User’s  Manual

SAFETY INSTRUCTIONS         1.Make sure your battery has enough voltage for the controller to recognize the battery type before first installation.

- 2.The battery cable should be as short as possible to minimize loss.

3.The regulator is only suitable for lead acid batteries: OPEN,AGM, GEL

it is not suited for nickel metal hydride, lithium ions or other batteries.

- 4.The charge regulator is only suitable for regulating solar modules. Never connect another charging source to the charge regulator.

- PRODUCT FEATURES

- 1.Build-in industrial micro controller.

- 2.LED display,auto memory function,IP57 protection.

- 3.Fully 4-stage PWM charge management.

- 4.Build-in short-circuit protection,open-circuit protection,reverse protection,over-load protection.

- 5.Dual mosfet Reverse current protection ,low heat production.

- SYSTEM CONNECTION

- Connect the battery to the charge regulator - plus and minus.it will auto adapt the battery type.

2.Connect the photovoltaic module to the regulator - plus and minus.

3. Connect the consumer to the charge regulator - plus and minus.

The reverse order applies when deinstalling!

An improper sequence order can damage the controller!

- SYSTEM MODE

The mode has been set inside the controller before exit the factory.Do ask  for further more information from your distributor.

| - mode | - description |
| --- | --- |
| - Light-open+delay | - Load on after sunset and closed after 1-13hour |
| - D2D | - Dusk to Dawn（have delay） |
| - System | - Load always output（unless battery is low） |

- TROUBLE SHOOTING

| - Situation | - Probable cause | - Solution |
| --- | --- | --- |
| - Charge LED not on when sunny | - Solar panel opened or reversed | - Reconnect |
| - Load LED off | - Mode setting wrong | - Set again |
|  | - Battery low | - recharge |
| - Load LED slow flashing | - Over load | - Reduce load watt |
| - Load LED slow flashing | - Short circuit protection | - Auto reconnect |
| - Power off | - Battery too low/reverse | - Check battery/connection |

- TECHNICAL PARAMETER

| MODEL | 5A-12V | 5A-12V-L | 5A-12V-8H |
| --- | --- | --- | --- |
| Batt voltage | 6V/12V auto adapt |  |  |
| Charge current | 5A |  |  |
| Discharge current | 5A |  |  |
| Max Solar input | 22V |  |  |
| Charge drop | <0.2V |  |  |
| Discharge drop | <0.25V |  |  |
| Equalization | 14.8V |  |  |
| Bulk | 14.5V |  |  |
| Acceptance | 14.2V PWM |  |  |
| Float | 13.8V |  |  |
| Charge reconnect | 13.0V |  |  |
| Discharge stop | 11.2V |  |  |
| Discharge reconnect | 12.6V |  |  |
| Self-consume | <10mA |  |  |
| Voltage of open light | Solar panel 8V（delay 10s） |  |  |
| Voltage of close light | Solar panel 8V（delay 60s） |  |  |
| Temperature compensation | N/C |  |  |
| Operating temperature | -35~+60 ℃ |  |  |
| Size/Weight | 60x25x18mm/50g |  |  |

*all red color voltage /2 while using 6V system.

*Product specifications are subject to change without prior notice.